# ICAR Difference Engine Number 3

Babbage was commissioned by the British government to provide accurate Log, Trig and other tables for naval navigation. His Difference Engine No. 1 (DE1) was started in 1822, however, funding later withdrawn due to increasing costs. He redesigned this machine during 1847 – 1849, producing Difference Engine No. 2 (DE2). However, Babbage only completed partial prototypes, he did not produce a fully working machine (later completed by the British Science museum).

DE2 evaluated polynomials using the method of finite differences, e.g. to repeatedly calculate the values of y = x3 we would manually calculate the first five values, taking the repeated difference of these values as shown in the left table of figure 1. This produces a set of initial coefficients i.e. 0, 1, 6, 6, and 0. Using the DE2 accumulator based architecture as shown in figure 2, these coefficients can be used to regenerate increasing values of x, as shown in the right table of figure 1.

Figure : calculating the initial coefficients for y = x3

ACC1

ACC2

ACC3

Figure 2 : DE2 architecture

For more information on this algorithm and DE2 refer to:

http://ed-thelen.org/bab/bab-intro.html

In general a computer’s architecture is independent of the technology used to implement it, however, there may be practical limitations due to size, cost, power etc. Therefore, the simplified architectural block diagram for DE2 shown in figure 3 can now be implemented in silicon i.e. the new and improved, Difference engine mark 3 (DE3).

Figure 3 : Simplified block diagram

To implement this architecture’s hardware a Field Programmable Gate Array (FPGA) will be used i.e. user configurable hardware. For more information on FPGA devices refer to:

http://en.wikipedia.org/wiki/Field-programmable_gate_array

To define this architecture’s hardware units a textual VHDL description will be used. VHDL is a double acronym: V standing for very high speed integrated circuit (VHSIC), HDL for hardware description language. This language was commissioned by the American Department of Defence in the early 1980’s as an implementation independent method of describing electronic systems, especially hardware. The DE3 hardware is defined in a number of text file as shown in figure 4.

Figure 4 : VHDL implementation

For more information on the VHDL language refer to:

http://en.wikipedia.org/wiki/VHDL

Like the original DE2, this new implementation only performs the algorithm of the method of finite differences. Initial coefficient values are passed through generic parameters as shown in the left panel of figure 4. When initialised these text files are then synthesized into a configuration bit file that sets up the required internal FPGA connections to realise the DE3 architecture in silicon. The RTL schematic (circuit diagram) and the FPGA floor plan (layout of the hardware elements) for this architecture are shown in figure 5.

Figure 5 : RTL schematic and FPGA Floor plan

Note, the eight repeated accumulator (ACC) units can be seen in the top of the schematic, also an idea of the approximate size of this implementation can be gained from the floor paln layout. Using this FPGA there is more than enough space for multiple difference engines to be implemented on the same FPGA. To display the results from the DE3’s output register an additional RS232 interface has been added to this design. This converts the output register’s 104bit result into a hexadecimal ASCII representation that can be transmitted to a PC. For more information on this interface refer to:

http://en.wikipedia.org/wiki/ASCII

http://en.wikipedia.org/wiki/RS-232

Using a terminal program running on a PC the data values generated by the DE3 can be displayed to the user as shown in figure 6. These data values are displayed as 32 digital hexadecimal values i.e. base 16.

Figure 6 : DE3 results for y = x3

Converting these hexadecimal values into decimal values we can see that we have regenerated the y = x3 values as defined in figure 1.

000016 = 000010

000116 = 000110

000816 = 000810

001B16 = 002710

004016 = 006410

007D16 = 012510

00D816 = 021610

015716 = 034310

...

The complete VHDL description for DE3 can be downloaded from the module web page.